bibliographic Information ツ

Author(s):

Zhong Y, Wang R, Feng H, Chen Y.

Title of article or book (if a chapter in a book, put the chapter # and title here, and put the title of the book in “Title of publication”):

International Journal of Advanced Robotic Systems. 2019

Title of publication, including Vol., Num., date (if article in magazine, newspaper, or journal):

Analysis and research of quadruped robot’s legs: A comprehensive review

Page numbers (if article in magazine, newspaper, or journal):

Publisher:

Sage Journal

Date published:

May 6, 2019

URL (if applicable):

https://journals.sagepub.com/doi/full/10.1177/1729881419844148

MLA citation:

1.

Zhong Y, Wang R, Feng H, Chen Y. Analysis and research of quadruped robot’s legs: A comprehensive review. International Journal of Advanced Robotic Systems. 2019;16(3). doi:10.1177/1729881419844148

Direct quotes: Include Page #s (For each quote, provide some context and/or analysis; you may also want to re-state the information in your own words. Quotes from your sources, and your understanding of them, will make up the bulk of your literature review)

- “As an important basic component of quadruped robots, mechanical legs provide the robots with excellent maneuverability and versatility, which determine the core application performance such as job adaptability, walking speed, and load capacity. A large number of robotics institutes for the last few decades have studied mechanical legs used by quadruped robots and published many research results. In this article, we collect these research results and classify them into three categories (prismatic legs, articulated legs, and redundant articulated legs) according to the degrees of freedom and then introduce and analyze them. On this basis, we summarize and study the design methods of the actuators and mechanical leg structures. Finally, we make some suggestions for the development of quadruped robot’s legs in the future. The motivation of this review is to summarize and analyze previous research efforts and provide useful guidance for future robotic designers to develop more efficient mechanical legs of quadruped robots.”

- “As a kind of legged robot, the quadruped robot has better load and high stability than the biped robot and has larger leg movement space, less mechanism redundancy, and less complexity than the multi-legged robot.4 Just as the famous Japanese foot robot research scholar Shigeo Hirose believes that the quadruped robot is the best form of legged robots from the aspects of stability and control difficulty and manufacturing cost. Therefore, the development of quadruped robots has always been highly valued, and it is of great theoretical and practical value to carry out relevant theoretical and technical research in depth.”

- “The design method of the prismatic leg was successfully applied in the 1980s and maintained the speed record of the quadruped robot at that time. This type of leg simulates the bounce of animal’s legs by a simple leg structure. It consists mainly of a rotating joint and a linearly moving prismatic joint.”

- “Compared with prismatic legs, the articulated leg uses a rotating joint instead of a linear prismatic joint to achieve leg length control. It has a similar function to the knee or elbow joint of the animal leg and has good biomimetic characteristics.”

- “Mechanical leg motion involves repeated dynamic events such as impact, rapid leg swing, and high force interaction with uncertain terrain. Designing actuator systems for high dynamic legged robots has always been one of the major challenges in robotics research. The ideal actuator is to maximize torque, bandwidth, and power while minimizing friction, inertia, and mass loss.41 However, due to the multifactor coupling relationship between them, it is not clear how to design an actuator to meet these contradictory requirements.”

- “Conventional electromagnetic actuators are often combined with gearbox to utilize high gear ratio to ensure maximum positional accuracy and stiffness. However, increasing the gear ratio will amplify the impact loads and increase the overall mechanical impedance of the system, while reducing the high gear ratio will increase friction losses and reduce the overall mechanical strength of the system. When the mechanical leg interacts quickly with the environment or there is an impact, a very high impact load is generated at the end actuator and the force control capability is limited. Conventional actuators with high gear ratio do not reduce the effects of the high mechanical impedance of the actuator system resulting from the high impact load. In order for a quadruped robot to dynamically interact with the ground and walk steadily in an unstructured environment, minimizing mechanical impedance is critical. To solve this problem, two new paradigms of actuator design have been proposed, namely the SEA and the PA.”

- “after decades of development, the mechanical legs have made great progress, but there is still a big gap compared with animal legs. The animal legs use the bone as a lever and the joint as a hub, driven by muscle contraction. The equal strength distribution of the leg bones, structural anisotropy, and the pore tube structure give it the lightest structural weight under specific load conditions. The power density of the muscle (0.3 kW/kg) is much smaller than the power density of electromagnetic actuator (5.68 kW/kg) and the hydraulic actuator (5.7 kW/kg).53 However, it can achieve better driving performance than electromagnetic and hydraulic actuators and support the body to achieve high-speed maneuvering capability such as running and jumping. This is inseparable from the special tensegrity structure of the muscle-skeleton. Faced with complex terrain and multi-tasking requirements, the wheel-leg hybrid drive system allows quadruped robots to have better terrain adaptability and motion diversity. Therefore, from the perspective of structural bionics, functional bionics, and wheel-leg hybrid drive, several suggestions for the development of the quadruped robot’s legs in the future are proposed.”

- “On flat terrain, the wheeled system has the advantages of high speed, outstanding performance in terms of payload-to-weight ratio, stable operation, and easy control. While legged system can perform tasks in rugged terrain. The wheel-leg hybrid drive system combines the characteristics of the wheel and leg system to design a quadruped robot’s leg, which combines the advantages of both forms of motion, that is, walking is used in rough-terrain and rolling is used in flat terrain. This design method improves the running efficiency of the quadruped robot. The application of the wheel-leg hybrid drive system is rarely found in quadruped robots. The robot handle developed by Boston Dynamics uses the wheel-leg hybrid drive system. By combining the rough-terrain capability of legs with the efficiency of wheels, Handle has the best of both worlds.”

Paraphrased Notes: Include Page #s (This is a space for you to track the flow of information, and the ways in which the article engages with other works.)

- Bionics invented in 1960

- Goal is to mimic real life

- Quadrupeds are the best complexity to output you can have

- Types of legs:

  - Prismatic: Hip joint(s) with a linear knee?

  - Articulated: Fully rotational, the ones like spot, and can be “mammal” type or “sprawling” type

- At least im not the only one to struggle with these STUPID MOTORS

- Yeah inertia sucks

- Apparently wheel legs are the way to go, also HOW ARE WE THAT WEAK?? Thats really cool that we have such weak limbs and yet we have evolved to get such an advantage.

Summary of Source (2-4 sentences)

This goes in detail about the design of quadruped legs, talking about all different forms, and how they are different. It also goes on to talk about changes that can be made, and where legs can be improved.

Does this help me answer my question? Why or why not?

Yes, it talks about the most critical part of quadrupeds.

Lingering Questions

How can we realize that joint-muscle build that they suggest in real life?

Connections to other sources

Legs.