bibliographic Information ツ

Author(s):

B. Katz, J. D. Carlo and S. Kim

Title of article or book (if a chapter in a book, put the chapter # and title here, and put the title of the book in “Title of publication”):

2019 International Conference on Robotics and Automation (ICRA)

Title of publication, including Vol., Num., date (if article in magazine, newspaper, or journal):

Mini Cheetah: A Platform for Pushing the Limits of Dynamic Quadruped Control

Page numbers (if article in magazine, newspaper, or journal):

6295-6301

Publisher:

IEEE

Date published:

12 August 2019

URL (if applicable):

https://ieeexplore.ieee.org/abstract/document/8793865

MLA citation:

B. Katz, J. D. Carlo and S. Kim, "Mini Cheetah: A Platform for Pushing the Limits of Dynamic Quadruped Control," 2019 International Conference on Robotics and Automation (ICRA), Montreal, QC, Canada, 2019, pp. 6295-6301, doi: 10.1109/ICRA.2019.8793865

Direct quotes: Include Page #s (For each quote, provide some context and/or analysis; you may also want to re-state the information in your own words. Quotes from your sources, and your understanding of them, will make up the bulk of your literature review)

- “At 9 kg, Mini Cheetah can be easily handled by a single operator. It features modular actuation designed for high-bandwidth torque control, torque density, and robustness to high-speed impacts. Despite its small size and low cost, Mini Cheetah has demonstrated highly dynamic behaviors including trot, trot-run, bounding, and pronking gaits at speeds up to 2.45 m/s−1, and 360° backflips from standing.” p6295

- “The mechanical design and actuation approach taken with the Mini Cheetah largely overlaps with its larger relative, the MIT Cheetah 3 [5]. It features a large range of motion on all degrees of freedom, low-inertia limbs, and backdrivable actuators, with no need for torque or force sensors or series compliance. Mini Cheetah is roughly 60% the length of Cheetah 3, with 0.21 meter and 0.19 meter link-lengths for the upper and lower links respectively, and 0.38 meter between the front and back legs. While Cheetah 3 has highly integrated actuation with custom electric motors and transmissions, Mini Cheetah has been designed for modularity and low cost.” p6295

- “The robot’s four identical legs were designed to maximize range of motion while minimizing limb mass and inertia. The actuators have sufficient internal bearings for the three degrees of freedom to be serially attached with no additional support structure, which would add weight and limit range of motion. The hip and knee motors are located co-axially at the hip, to minimize the moment of inertia. Torque is transmitted to the kneejoint through a Gates Poly Chain belt transmission which provides an additional 1.55:1 gear-up. Similar to the roller chain knee transmission in Cheetah 3, the belt allows ± 155 o range of motion from fully extended, so the robot can operate in both knee-forward and knee-backward configurations. The belt transmission does reduce torque control bandwidth at the knee by introducing a 30 Hz belt-actuator resonance. So far, we have not observed the belt compliance to affect locomotion performance, but if necessary the belt width can be easily increased to improve stiffness, at little weight or size penalty to the robot.” p 6296

- “The abduction/adduction (ab/ad) joint can rotate ± 120o, the hip joint ± 270o (limited by wire length), and the knee joint ± 155 0 ; see Figure 3. This range of motion will allow the robot to operate identically forwards, backwards, or upside-down, roll its body by 90o to fit through narrow gaps, and climb obstacles much taller than its leg length.” p 6296

- “Locomotion and other high-level control tasks are executed on an UP Board low-power single board computer with a quad-core Intel Atom CPU and 4 GB RAM, running Linux with the CONFIG-PREEMPT-RT patch for sudo-realtime operation. High level communication and data logging is accomplished using Lightweight Communication and Marshaling (LCM) [7]. The robot has sufficient internal space and power supply capability to add more computing as needed, for processing vision information for example, and LCM will allow easy integration of additional sensor data. The computer communicates with the 12 actuators through a custom quad CAN bus interface. The control, state estimation, and actuator communication loops run at 1 kHz, although our locomotion control typically does not run every loop iteration. Control code and simulation environment are largely shared with the Cheetah 3 Robot, which allows for easy transfer of control strategies between the two machines.” p 6296

- “To minimize the axial length of the actuator, the planetary gear set is placed inside the center of the stator. The hardened pins which support the planet needle roller bearings extend through the output of the actuator, and serve as locating and torque transmission features to the output. The planetary transmission uses all stock gears made by Misumi, and as a result has roughly 0.3o of backlash at the output. Whereas many robotics applications require high position accuracy, and therefore low backlash transmission, through our work with the MIT Cheetah robots we have not observed backlash to be of particular importance for legged locomotion.” p 6297

Paraphrased Notes: Include Page #s (This is a space for you to track the flow of information, and the ways in which the article engages with other works.)

I am going to skip notes purely because I have done essentially this article before and it would not be helpful to me.

Summary of Source (2-4 sentences)

The MIT mini cheetah is a “low cost” quadruped, designed for the rapid upgrade of quadrupeds. It is essentially a scaled down cheetah 3, and has some interesting features that make it different from other quadrupeds, notably its range of motion

Does this help me answer my question? Why or why not?

Yes, this is the best cheapest smallest quadruped ever made.

Lingering Questions

Why is it seemingly so easy for me to make a better version of this for so much cheaper?

Connections to other sources