SL1:

Hello everybody, welcome to Meridian Academy’s first ever Engineering JRPS. My name is Rey Banzaert, and welcome to my presentation on:

SL2:

How SLAM Functions on Quadrupedal Robots, and How It Impacts Their Design. I know that title means nothing to a lot of you and this is a dense topic, but I plan to keep this as fun as possible.

SL3:

This presentation is split into 4 different parts:

- Choosing a question

- What is SLAM?

- What's a quadruped?

- and actual JRPS and conclusion

SL4:

You may be asking yourselves, “Rey, how did you pick your question?????”

Well, as many of you know, I really, really like to build things. For the past two years, I’ve undertaken some pretty ridiculous projects, like teaching a fish to drive a car, and building an incredibly precise CNC router (both of these projects from scratch). This passion for robotics caused me to design my JRPS around my want to build a robot.

SL5:

My top 3 choices were: (click the thing)

1. Wanting to build a plane capable of VTOL  (click the thing)

2. Designing a commercial supersonic plane (click the thing)

3. And building a quadruped

As you may be able to tell, at this point I was really interested in planes. However, I still needed to narrow down to just one.

SL6:

So Rey, how did you do that? Well, its all from one quote.

SL7:

“No Rey, you can not build a plane for your JRPS” - Emily “the JRPS” Naito

SL8:

Over the summer, I interned with Professor John J. Leonard under the advisement of Kurran Singh at CSAIL, MIT — learning SLAM on the Jackal UGV by Clearpath Robotics (which can be seen in the bottom left). These two taught me everything I know about SLAM and ROS and just so much, they are two of the smartest minds in SLAM, and I cannot thank them enough. Additionally, working at MIT rekindled my love for quadrupedal robots, as my desk was right next to one. Turns out, across campus, was Professor Sangbae Kim, someone who I had met numerous times before, who turns out, essentially invented the quadrupedal robot with the MIT Cheetah series. One of his former students, Ben Katz, was one of the brilliant minds behind the MIT Cheetah 3 and MIT Mini Cheetah, and he would eventually become my pseudo-interviewee.

At the end of the summer, I was super interested in both SLAM and quadrupeds — resulting in me wanting to build a quadruped capable of SLAM. The other questions were only there because Betsy said I needed 3.

SL9:

Alright, enough stalling, lets get into this. SLAM, which stands for Simultaneous Localization and Mapping (yes I know this isnt much better), is the field that asks the question, “given a robot dropped in a random environment, how does it both build a map of its environment, and figure out where it is on that map.” For us humans, this is incredibly easy, and we do it every day. Take walking, for example. Subconsciously, you are scanning your environment, planning path for you to go through in said environment, telling your legs where to go, and retaining that information about the environment for later use. And so much more can happen, like you tripping — How do you catch yourself? Say a kid runs out in front of you — How do you stop in time to not hit the kid?

Additionally, for those of you who have taken Emily’s Engineering class (wow Emily teaches engineering she would be a great fit for me right guys???), any moving robot will not do exactly what you tell it to do because the real world sucks, and therefore we can never know exactly where the robot is, resulting in the problem we call dead reckoning. SLAM also needs to counteract this.

SL10:

Lets take an example. Lets say you are the Jackal I worked on over the summer, and you have to get through this maze, with only (click the thing) a LIDAR and wheels. This is pretty easy for us, as we can see the whole maze, and plan a path through. However, the only thing the robot can see is something that looks like

SL11: This, a set of depth data at certain angles from the LiDAR. This data can be turned into a known object that from the top looks like this (click). This is a good wall for us to have, but heres where the problems start to occur. Because its from a single point —the lidar— the data isnt flat, meaning that the object actually looks like this (click). And even thats wrong because the lidar is not at the dead center of our robot, and some of these data points may be wrong, and so on. However, by doing a bunch of gross math (please feel free to talk to me if you would like to learn more), we can get usable data. However, as you can see, this sucks, and involves way too many steps, to get just 1 piece of our puzzle. In order to help with this problem, roboticists created

SL12:

ROS, or the Robotic Operating System (which is not an operating system), is a framework of nodes, topics, and preconfigured packages to allow for a robot to run easily. In normal people terms, it allows for you to send all the data from different sources to different “topics”, and then have little individual programs called “nodes” do different things with said data. Here, you can see the ROS navigation stack, commonly referred to as the Nav stack, which takes in data through a bunch of external topics, like the sensor data, odometry, and maps, and outputs where the robot should move. The internals of the Nav stack can then be substituted for a variety of programs that can essentially be drag-and-dropped into it. This allows for the use of hundreds of different ways of mapping, path planning, obsticle avoidance, and anything you could ever want, with minimal effort on the user's end. This allows for mapping software like gmapping, on the top left, which is a way of 2d mapping that makes a grid of “safe” and “not safe” squares. The dark squares are walls, the white squares are safe areas, and that gray/green color is unmapped areas. In addition, you can use 3d mapping software like RTAB-MAP, on the top right, which takes 3d data, and reconstructs a scene, creating “safe” and “not safe” zones. Each of those points on the RTAB map has an XYZ and a color, but I can’t really show that. Along with mapping softwares, the Nav stack allows for the implemtation of exploration packages, like m-explore, to tell the robot to go explore its environment.

SL13:

Okay finally quadruped time. Quadruped robots are, in the simplest term, robot dogs. There are some you may recognize, like:

- MIT Cheetah 2

- MIT Cheetah 3

- MIT Mini Cheetah

- BigDog

- AnyMal

- Unitree Go2

- Lite3

- HyQ

- KAIST HOUND

- And of course, the Boston Dynamics Spot robot

SL14:

Quadrupeds usually consist of 3 parts: (click)

- The body, which is a frame to hold everything together, and is usually where the batteries and computers go (click)

- The legs, which are legs (click)

- The actuators, which provide power to the legs.

SL15:

The body is pretty hard to get wrong, all that is really needed is some way to hold the legs together. Ideally, this, and every part, should be as light as possible, as more weight requires more force and energy to move.

Legs are also pretty easy. Typically, legs have two joints (click) in this configuration, one at the knee and one at the hip. The only complication is that in order to make the leg as light as possible, putting the motor on the knee is not really an option. Putting the motor on the knee would result in the quadruped being way slower as the leg has a higher moment of inertia, which would really suck in say a professional setting *cough cough* notice the motors in the knees *cough cough* (click). In order to get the rotational force from the motors to the knees, typically a chain or belt is used, like the one on the image to the right, which represents a pretty typical leg design, with an actuator at the top, and a way for it to be held by other actuators.

SL16:

However, now we get to the gross stuff, how the quadruped actually gets powered. The actuators. Unlike the body and legs, the actuator is pretty hard to get right. While there are linear actuators, I am going to mostly focus on rotational actuators. Quadrupeds, as a type of robot, a relatively slow in terms of how fast each joint actually spins, and in exchange, requires a lot of rotational force, known as torque. This trade-off is the main separation between cars and quadrupeds, and while quadrupeds require more torque and overall more energy, they can access more places and be more agile than cars. Because of this need for high torque, low speed motors, the MIT Cheetah series, and therefore every robot after it, adopted QDD (quasi-direct drive) BLDC (brushless dc) actuators. These actuators, while stupidly expensive, provide a huge amount of torque for the power you give them, and can be very light. I wont get too into this, but in order to control these motors, FOC or field-oriented control controllers are used, turning dc motors into a better stepper motor (if you know great, if you dont know im sorry but i dont have the time to explain find me later).

SL17:

My experience going through JRPS was really rough. On top of having to figure out how to actually do an engineering JRPS, there were countless struggles along my path. (click)

My first half was struggling to write a lit review. (click)

My second half was wondering how you design a quadrupedal robot. (click)

My third half was designing a quadrupedal robot, (click)

And my fourth half was wondering why there are two of me (haha funny joke laugh)

SL18:

However, the grind and struggle was all worth it. Allow me to introduce:

SL19:

Leo the Leopard. Something you may note is that it doesnt look exactly like my mockups, and it also isnt walking. Because of some wonderful policies by our government, the places I was buying my motors and motor controllers are no longer shipping to the U.S., meaning I could not get all my motors and motor controllers. Additionally, because the shop I use to manufacture had the end of the semester, mixed with the waterjet breaking, a lot of what I wanted to make couldnt be made. However, there is 1 functioning leg (click), which performs better than anticipated, and I plan to get it fully operational over the summer. The one operational leg is made of ⅛” 5052 sheet aluminum, with a PLA insert and a PLA foot — with a 10 speed bike chain connecting the top of the leg to the knee. Every other leg has the same, except, replace the sheet metal with acrylic, as the water jet I was planning to use is currently out of service. Because of the out of service water jet, to simplify everything, (click) the two leg actuators are 3d printed, so they can’t actually spin. The actuators will eventually house an Eaglepower 90kv 8308 motors (which coincidentally is the amount of tears i shed on JRPS). Those are then connected to a 16:1 planetary gear box, with 16 being chosen for the ratio as its the amount of times I had to re-design the actuator from the ground up. The total length of the legs fully extended is 0.8 meters, but standing, its height comes in at 0.56 meters. The final hip actuator in each of those is made out of acrylic. (click) The frame is a hand-crafted masterpiece by my wonderful father dearest.

SL20:

Smashing everything together, throwing in some (click) Odrive v3 motor controllers, an (click) NVIDIA Jetson Nano, a (click) battery, and a Zed Mini Camera, you get Leo. Leos total gross weight comes in at just above 20kg, but that weight goes to good use, with a theoretic maximum payload of 15kg, and being able to run at a theorized 5m/s. Leo should have no problem jumping, going up stairs, handling rugged terrain, and, more importantly, may (heavy emphasis on the may) be able to do a backflip. With a raw material cost somewhere around ~$350, each actuator costing ~$70, the batteries costing $150, the camera costing $400, and the computer costing $100, the robot comes out to costing a total of ~$1840. While not being as low cost as I was hoping for, the robot is still ridiculously cheap for what it can achieve. However, cutting down on cost has hindered the robot in a few ways. The biggest way is that its battery life sucks, with it being able to walk for only an hour before needing to be recharged. Increasing battery capacity is the main thing I wish to improve on in the future, but that will cut its payload and increase its gross weight.

SL21: Using this platform, I plan to implement an open-source quadruped controller called CHVMP, which will allow me to control it with the previously mentioned Nav stack. I intend to use ORB-SLAM with the Zed Mini camera I just brought up, using its stereo vision to construct maps. I also hope to apply M-explore, a package which will allow for it to explore all on its own.

SL22: Because Emily says I have to, here’s my conclusion. SLAM is the future of autonomous robotics. The ability to have a robot do tasks by itself and self correct when it fails is revolutionary, and it has already shown its uses in many industrial settings. Quadrupeds capable of SLAM have already proven useful in search and rescue, industrial tasks, and working with highly toxic equipment. While again, still relatively new, if actuator design is able to significantly improve, they have the potential to be one of the greatest resources we have ever made.

SL23: Before I finish, I would like to give my thanks to a couple people. Firstly, I want to thank everybody at MIT, specifically the shop staff of Papalardo Lab, Bill, Scott, all of the Steves, Danny, and previously Tasker, for inspiring me and helping my ridiculous projects over the years, and treating me as their own. You all have allowed me to accomplish what I have, and I can not thank you enough. I would also like to thank Ben Katz and Sangbae Kim from the Biomimetics Lab, who have paved the way for quadrupeds and let me ask a million questions. I would also like to thank all of MRG@CSAIL, especially Hayden, my fellow intern, Alan, Jungseok, and especially Prof. John J. Leonard and Dr. Kurran Singh, who let me intern there and study under them.

I would also like to thank all of my fellow juniors for tolerating me on this journey. I know I've annoyed you all with my check-ins and whatnot, but being in a class with you all this year has been fabulous, and I can’t wait to spend my final year here with you all.

I would also like to thank my wonderful family, Steve Banzaert, Amy Banzaert, and Taff Banzaert, who have listened to my countless rants about the topic, have dealt with my project being strewn throughout our house, and just so many annoyances.

However, I would like to focus on my dad for a second. Dad, you have been there with me for every step in this process, being a constant source of invaluable information, a wonderful mind to brainstorm with, and someone who I could count on to tell me the truth. Your dedication, time, passion, and oh so much money have truly made this my greatest accomplishment, and I truly can not thank you enough. If anyone here ends up talking to him at any time in the future, please tell him he made a really good box.

And of course, the one person who I could not have done this without, my wonderful JRPS advisor, Emily Naito: who gave me the most wonderful comments on my lit review and presentation, who encouraged me to work through the most challenging problems I've ever dealt with, who endured my overwhelming stupidity, who tolerated me demanding that I build an absurdly big robot, and who bullied me relentlessly. Emily, we knew this day was coming for years, and yet this journey we have been on together is one I never expected to be so incredible in every way, shape, and form. Truly, from the bottom of my heart, thank you for everything.

And finally, for you, my audience,

SL25: Thank you for coming to my Ted Talk. Do you have any questions?